Computer Networks Lab (B18CS5080)

SCHOOL OF COMPUTING

AND

INFORMATION TECHNOLOGY

Submitted to: - prof. Farooque Azam sir and prof. Nikhil Tengli Sir

NAME: Manasa. M. A

SRN: R18CS212

SEM: 5th

SECTION: ‘D’

Program 1

Introduction to: (a) discrete event simulation, (b) NS3, (c) NS 3 Installation, (d) NetAnim.

- Introduction to Discrete event simulation

## System: A collection of entities that act and interact together toward the accomplishment of some logical end.

## Discrete system: State variables change instantaneously at separated point in time, e.g., a bank, since state variables - number of customers, change only when a customer arrives or when a customer finishes being served and departs

## Continuous system: State variable change continuously with respect to time, e.g., airplane moving through the air, since state variables - position and velocity change continuously with respect to time

## Why Simulation?

## Many systems are highly complex, precluding the possibility of analytical solution

## The analytical solutions are extraordinarily complex, requiring vast computing resources

## Thus, such systems should be studied by means of simulation

## What is Discrete-Event Simulation (DES)?

## A discrete-event simulationmodels a system whose state may change only at discrete pointmodels a system whose state may change only at discrete pointin time.

- Introduction to NS3

ns-3 is a discrete-event network simulator, targeted primarily for research and educational use. ns-3 is free software, licensed under the GNU GPLv2 license, and is publicly available for research, development, and use.

The goal of the ns-3 project is to develop a preferred, open simulation environment for networking research: it should be aligned with the simulation needs of modern networking research and should encourage community contribution, peer review, and validation of the software.

- NS 3 Installation

Download the ns3 package (ns-allinone-3.30) from

https://www.nsnam.org and follow following steps:
1. $cd Desktop
2. $cd ns-allinone-3.28
3. $./build.py --enable-examples --enable-test
4. $cd ns-3.28
5. $./waf --build-profile=debug --enable-examples --enable-test configure

once all the configuration is done NS-3 inastallation is completed.

To check whether NS-3 is successfully installed verify with this:
$./waf --run hello-simulator

The output will be shown as hello-simulator

- Introduction to NetAnim:

$ Sudo apt install qt4-default qt4-make

$ cd NetAnim

$ qmake NetAnim.pro

$ make

Execution of NetAnim

$ Cd to NetAnim from ns3.28...

$ ./NetAnim

Program 2

Write a NS3 program to connect two nodes with a point to pint link, which have unique interface. Analyze the network performance using UDP client server.

CODE:

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/netanim-module.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("FirstScriptExample");

int main (int argc, char *argv[])

{

CommandLine cmd;

cmd.Parse (argc, argv);

Time::SetResolution (Time::NS);

LogComponentEnable ("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable ("UdpEchoServerApplication", LOG_LEVEL_INFO);

NodeContainer nodes;

nodes.Create (2);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute ("DataRate", StringValue ("5Mbps"));

pointToPoint.SetChannelAttribute ("Delay", StringValue ("2ms"));

NetDeviceContainer devices;

devices = pointToPoint.Install (nodes);

InternetStackHelper stack;

stack.Install (nodes);

Ipv4AddressHelper address;

address.SetBase ("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer interfaces = address.Assign (devices);

UdpEchoServerHelper echoServer (9);

ApplicationContainer serverApps = echoServer.Install (nodes.Get (1));

serverApps.Start (Seconds (1.0));

serverApps.Stop (Seconds (10.0));

UdpEchoClientHelper echoClient (interfaces.GetAddress (1), 9);

echoClient.SetAttribute ("MaxPackets", UintegerValue (1));

echoClient.SetAttribute ("Interval", TimeValue (Seconds (1.0)));

echoClient.SetAttribute ("PacketSize", UintegerValue (1024));

pointToPoint.EnablePcapAll ("first");

ApplicationContainer clientApps = echoClient.Install (nodes.Get (0));

clientApps.Start (Seconds (2.0));

clientApps.Stop (Seconds (10.0));

AnimationInterface anim ("first.xml");

anim.SetConstantPosition(nodes.Get (0), 10.0, 10.0);

anim.SetConstantPosition(nodes.Get (1), 20.0, 30.0);

Simulator::Run ();

Simulator::Destroy ();

return 0;

}

OUTPUT:

Program 3

Write a NS 3 program to demonstrate bus topology. Analyze the performance using UDP based applications.

CODE:

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/ipv4-global-routing-helper.h"

#include "ns3/netanim-module.h"

// Default Network Topology

//

//       10.1.1.0

// n0 -------------- n1   n2   n3   n4

//    point-to-point  |    |    |    |

//                    ================

//                      LAN 10.1.2.0

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("SecondScriptExample");

int

main (int argc, char *argv[])

{

bool verbose = true;

uint32_t nCsma = 3;

CommandLine cmd;

cmd.AddValue ("nCsma", "Number of \"extra\" CSMA nodes/devices", nCsma);

cmd.AddValue ("verbose", "Tell echo applications to log if true", verbose);

cmd.Parse (argc,argv);

if (verbose)

{

LogComponentEnable ("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable ("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

nCsma = nCsma == 0 ? 1 : nCsma;

NodeContainer p2pNodes;

p2pNodes.Create (2);

NodeContainer csmaNodes;

csmaNodes.Add (p2pNodes.Get (1));

csmaNodes.Create (nCsma);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute ("DataRate", StringValue ("5Mbps"));

pointToPoint.SetChannelAttribute ("Delay", StringValue ("2ms"));

NetDeviceContainer p2pDevices;

p2pDevices = pointToPoint.Install (p2pNodes);

CsmaHelper csma;

csma.SetChannelAttribute ("DataRate", StringValue ("100Mbps"));

csma.SetChannelAttribute ("Delay", TimeValue (NanoSeconds (6560)));

NetDeviceContainer csmaDevices;

csmaDevices = csma.Install (csmaNodes);

InternetStackHelper stack;

stack.Install (p2pNodes.Get (0));

stack.Install (csmaNodes);

Ipv4AddressHelper address;

address.SetBase ("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign (p2pDevices);

address.SetBase ("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces;

csmaInterfaces = address.Assign (csmaDevices);

UdpEchoServerHelper echoServer (9);

ApplicationContainer serverApps = echoServer.Install (csmaNodes.Get (nCsma));

serverApps.Start (Seconds (1.0));

serverApps.Stop (Seconds (10.0));

UdpEchoClientHelper echoClient (csmaInterfaces.GetAddress (nCsma), 9);

echoClient.SetAttribute ("MaxPackets", UintegerValue (1));

echoClient.SetAttribute ("Interval", TimeValue (Seconds (1.0)));

echoClient.SetAttribute ("PacketSize", UintegerValue (1024));

ApplicationContainer clientApps = echoClient.Install (p2pNodes.Get (0));

clientApps.Start (Seconds (2.0));

clientApps.Stop (Seconds (10.0));

Ipv4GlobalRoutingHelper::PopulateRoutingTables ();

pointToPoint.EnablePcapAll ("second");

csma.EnablePcap ("second", csmaDevices.Get (1), true);

AnimationInterface anim ("second.xml");

anim.SetConstantPosition (p2pNodes.Get (0), 10.0, 10.0);

anim.SetConstantPosition (csmaNodes.Get (0), 20.0, 20.0);

anim.SetConstantPosition (csmaNodes.Get (1), 30.0, 30.0);

anim.SetConstantPosition (csmaNodes.Get (2), 40.0, 40.0);

anim.SetConstantPosition (csmaNodes.Get (3), 50.0, 50.0);

Simulator::Run ();

Simulator::Destroy ();

return 0;

}

Output:

Program 4

Write a NS 3 program to demonstrate star topology. Analyze the performance using UDP based applications.

CODE:

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/netanim-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/point-to-point-layout-module.h"

// Network topology (default)

//

//        n2 n3 n4              .

//         \ | /                .

//          \|/                 .

//     n1--- n0---n5            .

//          /|\                 .

//         / | \                .

//        n8 n7 n6              .

//

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("StarAnimation");

int

main (int argc, char *argv[])

{

//

// Set up some default values for the simulation.

//

Config::SetDefault ("ns3::OnOffApplication::PacketSize", UintegerValue (137));

// ??? try and stick 15kb/s into the data rate

Config::SetDefault ("ns3::OnOffApplication::DataRate", StringValue ("14kb/s"));

//

// Default number of nodes in the star.  Overridable by command line argument.

//

uint32_t nSpokes = 8;

std::string animFile = "star-animation.xml";

uint8_t useIpv6 = 0;

Ipv6Address ipv6AddressBase = Ipv6Address("2001::");

Ipv6Prefix ipv6AddressPrefix = Ipv6Prefix(64);

CommandLine cmd (__FILE__);

cmd.AddValue ("nSpokes", "Number of spoke nodes to place in the star", nSpokes);

cmd.AddValue ("animFile",  "File Name for Animation Output", animFile);

cmd.AddValue ("useIpv6",   "use Ipv6", useIpv6);

cmd.Parse (argc, argv);

NS_LOG_INFO ("Build star topology.");

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute ("DataRate", StringValue ("5Mbps"));

pointToPoint.SetChannelAttribute ("Delay", StringValue ("2ms"));

PointToPointStarHelper star (nSpokes, pointToPoint);

NS_LOG_INFO ("Install internet stack on all nodes.");

InternetStackHelper internet;

star.InstallStack (internet);

NS_LOG_INFO ("Assign IP Addresses.");

if (useIpv6 == 0)

{

star.AssignIpv4Addresses (Ipv4AddressHelper ("10.1.1.0", "255.255.255.0"));

}

else

{

star.AssignIpv6Addresses (ipv6AddressBase, ipv6AddressPrefix);

}

NS_LOG_INFO ("Create applications.");

//

// Create a packet sink on the star "hub" to receive packets.

//

uint16_t port = 50000;

Address hubLocalAddress;

if (useIpv6 == 0)

{

hubLocalAddress = InetSocketAddress (Ipv4Address::GetAny (), port);

}

else

{

hubLocalAddress = Inet6SocketAddress (Ipv6Address::GetAny (), port);

}

PacketSinkHelper packetSinkHelper ("ns3::TcpSocketFactory", hubLocalAddress);

ApplicationContainer hubApp = packetSinkHelper.Install (star.GetHub ());

hubApp.Start (Seconds (1.0));

hubApp.Stop (Seconds (10.0));

//

// Create OnOff applications to send TCP to the hub, one on each spoke node.

//

OnOffHelper onOffHelper ("ns3::TcpSocketFactory", Address ());

onOffHelper.SetAttribute ("OnTime", StringValue ("ns3::ConstantRandomVariable[Constant=1]"));

onOffHelper.SetAttribute ("OffTime", StringValue ("ns3::ConstantRandomVariable[Constant=0]"));

ApplicationContainer spokeApps;

for (uint32_t i = 0; i < star.SpokeCount (); ++i)

{

AddressValue remoteAddress;

if (useIpv6 == 0)

{

remoteAddress = AddressValue(InetSocketAddress (star.GetHubIpv4Address (i), port));

}

else

{

remoteAddress = AddressValue(Inet6SocketAddress (star.GetHubIpv6Address (i), port));

}

onOffHelper.SetAttribute ("Remote", remoteAddress);

spokeApps.Add (onOffHelper.Install (star.GetSpokeNode (i)));

}

spokeApps.Start (Seconds (1.0));

spokeApps.Stop (Seconds (10.0));

NS_LOG_INFO ("Enable static global routing.");

//

// Turn on global static routing so we can actually be routed across the star.

//

if (useIpv6 == 0)

{

Ipv4GlobalRoutingHelper::PopulateRoutingTables ();

}

// Set the bounding box for animation

star.BoundingBox (1, 1, 100, 100);

// Create the animation object and configure for specified output

AnimationInterface anim (animFile);

pointToPoint.EnablePcapAll ("star-animation");

NS_LOG_INFO ("Run Simulation.");

Simulator::Run ();

Simulator::Destroy ();

NS_LOG_INFO ("Done.");

return 0;

}

OUTPUT:

Program 5

Install NMAP, and execute atleast 5 commands to demonstrate the scanning of networks hosts and ports.

1. Basic Nmap Scan against IP or host

nmap www.reva.edu.in

Now, if you want to scan a hostname, simply replace the IP for the host, as you see below: nmap www.reva.edu.in

2. Scan specific ports or scan entire port ranges on a local or remote server

nmap -p 1-50www.google.com

In this example, we scanned all 50 ports for www.google.com

3. Scan multiple IP addresses

Scan multiple IP addresses. For this you need to use this syntax:

nmap 1.1.1.1 8.8.8.8

Scan consecutive IP addresses:

nmap -p 1.1.1.1,2,3,4

This will scan 1.1.1.1, 1.1.1.2, 1.1.1.3 and 1.1.1.4.

4. Scan the most popular ports

Using “–top-ports” parameter along with a specific number lets you scan the top X most common ports for that host, as we can see:

nmap --top-ports 5www.google.com

5. Scan using TCP or UDP protocols

Nmap works for both TCP and UDP protocols. And while most services run on TCP, we can also get a great advantage by scanning UDP-based services. Let’s see some examples.

Standard TCP scanning output:

nmap -sT www.reva.edu.in

nmap -sU 192.168.1.1